OE0220 2相混合式步进电机驱动器

OE0220  2 Phase Stepper Motor Driver   <Suit for NEMA8-NEMA17 Stepper Motor>

概述  Descriptions

OE0220步进电机细分驱动器是一款高性价比的两相步进电机驱动器 ,最大可提供2.0A的电流输出，直流电源输入。本款驱动器采 用先进的纯正弦波电流控制技术，具有大扭矩，低噪声，低振动等特点,适用于尺寸为28，35，39，42等各类2相或4相混合式步进电机.

OE0220 is a type of 2 phase hybrid Stepper motor driver . this driver input DC 12-DC36 Voltage. This kind of drive with advanced pure sine wave current control technology, with large torque, low noise, low vibration and other characteristics, is suitable for the size of 20, 28,35,39,42 and other kinds of phase 2 or 4 phase hybrid stepping motors.

特点Features

*平均电流控制，两相正弦电流输出

*直流12~36V,交流12~24V 供电

*光电隔离信号输入、输出

*有过压﹑欠压﹑过流﹑相间短路保护功能

*八档细分和自动半流功能

*八档输出相电流设置

*可驱动任何2.0A相电流以下两相、四相混合式步进电机 
*20KHz斩波频率

*Average current control,2 phase sinusoidal output current drive

*supply voltage from 12VDC~36VDC and 12VAC~24VAC

*Opto-isolated signal I/O

*Overvoltage, under voltage,overcurrent,phase short cicuit protection

*8 channels subdivision and automatic idle-current reduction

*8 channels output phase current setting

* Can drive 2 phase and 4phase hybrid stepper motor when phase current less than 2.0A

*20KHz PMW.

性能指标

输入电压    直流12~36V

输出电流    0.3A~2.0A

温    度    工作温度-10°~45°，存放温度-20°~125°

环境湿度    不凝露，不结霜

气    体    禁止有可燃气体和导电灰尘

重    量    180克

Mechanical Specification

Input Voltage     12~36V DC,  12~24V AC

Output current    0.3A~2.0A

Temperature      working temperature -10°~45°,stocking temperature ~20°~70°

Humidity          Not condensation , no water droplets

Gas              Prohibition of combustible gases and conductive dust

Weight            180g

拨码开关 Switch Choice:

1、细分选择Microstepping choice:

| SW5 | On | Off | On | Off | On | Off | On | Off |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW6 | On | On | Off | Off | On | On | Off | Off |
| SW7 | On | On | On | On | Off | off | Off | Off |
| Micro | 1 | 2 | 4 | 8 | 16 | 32 | 64 | 128 |

2、电流选择Current choice：

| SW1 | On | Off | On | Off | On | Off | On | Off |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SW2 | On | On | Off | Off | On | On | Off | Off |
| SW3 | On | On | On | On | Off | Off | off | Off |
| Current（A） | 0.3 | 0.4 | 0.5 | 0.6 | 1.0 | 1.2 | 1.5 | 2.0 |

3、安装尺寸installation dimension