OE2M2283  2相数字驱动器

OE2M2283 2 Phase Stepper Motor Driver

概述  Descriptions

OE2M2283是采用等角度恒力矩细分型高性能步进电机驱动器，适用130mm、110mm等8A以下两相、四相混合式步进电机。该驱动器采用类似伺服控制原理的电路，此电路具有运行平稳、噪音低、振动低、电机温升低等特点。因驱动器工作电压高，使电机在高速时力矩大大的高于其他两相、四相混合式及传统式步进电机。其微步细分有16种，最大细分数1/256（步数为51200步/转）；其工作电流范围为2.0A-8.3A，输出电流共有16档，电流分辨率约为0.4A；具有自动半流、自检、过压、欠压和过流等保护功能。本驱动器为交流供电，建议工作电压范围为150VAC-220VAC，电压不超过240VAC不低于130VAC。

OE2M2283 is to use the perspective of constant torque subdivided type high-performance stepper motor drives, for 130 mm and 110 mm, 8 a two phase, four phase hybrid stepping motors.The driver adopted similar servo control principle of the circuit, the circuit has stable operation, low noise, low vibration, low motor temperature rise, etc.Due to the high working voltage, drive torque motor in high speed greatly higher than the other two phase, four phase hybrid and traditional stepper motor.Its micro step has 16 kinds of segment, the maximum fine fraction 1/256 step (step 51200 / roll);The scope of its working current of 2.0 A - 8.3 A, output current, A total of 16 file current resolution of about 0.4 A;With automatic flow, self-check, over-voltage, under-voltage and over-current protection function.This drive for the ac power supply, it is recommended that the operating voltage range is 150 vac - 220 vac, voltage does not exceed 240 vac is not lower than 130 vac.

特点Features

- 高性能、低价格

- 内置微步细分

- 停止时自动半流

- 光电隔离信号输入/输出

- 脉冲响应频率最高可达200KHz

- 低温升、低振动

- 在线自适应PID技术

- High-performance, low price

- micro-step

- Automatic idle-current reduction

- Optical isolating signal I/O

- Max response frequency up to 200Kpps

- Low temperature rise, smooth motion

- Online adaptive PID technology

应用Application

适合各种中小型自动化设备和仪器，例如：雕刻机、打标机、切割机、激光照排、绘图仪、数控机床、自动装配设备等。在用户期望小噪声、高速度的设备中应用效果特佳。

Suitable for all kinds of small and medium sized automation equipment and instruments, such as: engraving machine, marking machine, cutting machine, laser lighting, plotter, cnc machine, automatic assembly equipment, etc. It use very well in the equipment that user expects small noise, and high peed

电气指标和工作环境 Electrical, mechanical and environmental indicators

| 电气参数 | 最小值 | 典型值 | 最大值 | 单位 |
| --- | --- | --- | --- | --- |
| 输入电压交流(VC) | 150 | - | 220 | VAC |
| 输出电流峰值 | 0 | - | 8 | A |
| 脉冲频率 | 0 | - | 200 | KHZ |
| 输入信号电流 | 7 | 10 | 16 | mA |

使用环境及参数

| 冷却方式 |  | 自然冷却或强制风冷 |
| --- | --- | --- |
| 使用环境 | 场合 | 不能放在其它发热的设备旁，要避免粉尘、油雾、腐蚀性气体，湿度太大及强振动场所，禁止有可燃气体和导电灰尘； |
|  | 温度 | -5℃ ～ +50℃ |
|  | 湿度 | 40 ～ 90%RH |
|  | 振动 | 5.9m/s2MAX |
| 保存温度 |  | -20℃～80℃ |
| 使用海拔 |  | 1000米以下 |
| 重量 |  | 约 1.7千克 |

Electrical indicators

| Parameter | Min | Typical | Max | Unit |
| --- | --- | --- | --- | --- |
| Input Voltage(AC) | 150 | - | 220 | VAC |
| Output current | 0 | - | 8.3 | A |
| Pulse Signal Frequency | 0 | - | 200 | KHZ |
| Logic Signal Current | 7 | 10 | 16 | MA |

Using environment and parameters

| Cooling method |  | Natural cooling or forced air cooling |
| --- | --- | --- |
| Using environment | occasion | Can not be placed next to other heating equipment, should avoid dust, oil mist, corrosive gas, too big humidity and strong vibration place, prohibit to have combustible gas and conductive dust |
|  | temperature | -5℃ ～ +50℃ |
|  | humidity | 40 ～ 90%RH |
|  | vibration | 5.9m/s2MAX |
| Storage temperature |  | -20℃～80℃ |
| Using altitude |  | Below 1000 metres |
| Weight |  | About 1.7kg |

3、安装尺寸installation dimension